EXHIBIT “A”

PROPOSED REVISION

Allegheny County Health Department Rules and Regulations

Article XV, Plumbing and Building Drainage

§718 Rehabilitation of Building Sewers and Building Drains Insights

ALLEGHENY COUNTY HEALTH DEPARTMENT

RULES AND REGULATIONS

Deletions are shown with strikethroughs.

Additions are shown in larger font, bolded, and underlined.

Table of Contents should also be modified and changed accordingly and represented in larger, bold and underlined text.

ARTICLE XV – PLUMBING AND BUILDING DRAINAGE

AC-Photo-initiated cure. A curing process in cured-in-place pipe (CIPP) rehabilitation where a resin liner is exposed to light of a specific wavelength, which activates chemical polymerization that ultimately transforms liquid resin into a solid structural pipe within the original host pipe.

AC-PLUMBER (MASTER). The term “Master Plumber” shall mean a person other than a

journeyman or apprentice plumber, who is not on inactive status, having satisfied the

Allegheny County Health Department as to his knowledge of the installation of plumbing

and has been licensed by the Department to install plumbing as a Master Plumber.

PLUMBING. The practice, materials and fixtures utilized in the installation, maintenance,

extension and alteration of all piping, fixtures, plumbing appliances and plumbing

appurtenances, within or adjacent to any structure, in connection with sanitary drainage or

storm drainage facilities; venting systems; and public or private water supply systems.

PLUMBING APPLIANCE. Any one of a special class of plumbing fixtures intended to

perform a special function. Included are fixtures having the operation or control dependent

on one or more energized components, such as motors, controls, heating elements, or

pressure- or temperature-sensing elements.

Such fixtures are manually adjusted or controlled by the owner or operator, or are operated

automatically through one or more of the following actions: a time cycle, a temperature

range, a pressure range, a measured volume or weight

Chapter 7 Sanitary Drainage

717.7 Installation. The installation of relining materials shall be performed in accordance

with the manufacturer's installation instructions, applicable referenced standards and this

code.

717.7.1 Material data report. The installer shall record the data as required by the relining

material manufacturer and applicable standards. The recorded data shall include but is not

limited to the location of the project, relining material type, amount of product installed

and conditions of the installation. A copy of the data report shall be provided to the code

official prior to final approval.

717.8 Post-installation recorded video camera survey. The completed, relined piping

system shall be inspected internally by a recorded video camera survey after the system

has been flushed and flow-tested with water. The video survey shall be submitted to the

code official prior to finalization of the permit. The video survey shall be reviewed and

evaluated to provide verification that no defects exist. Any defects identified shall be

repaired and replaced in accordance with this code.

717.9 Certification. A certification shall be provided in writing to the code official, from the

permit holder, that the relining materials have been installed in accordance with the

manufacturer's installation instructions, the applicable standards and this code.

717.10 Approval. Upon verification of compliance with the requirements of Sections 717.1  through 717.9, the code official shall approve the installation.

SECTION 718

REHABILITATION OF BUILDING SEWERS AND BUILDING DRAINS INSIGHTS

AC-718.1 Cure-in-place. Sectional cure-in-place rehabilitation of building sewer

piping and sewer service lateral piping shall be in accordance with ASTM F2599.

Photo-initiated curing shall be used for the rehabilitation of building

sewer piping by cured-in-place pipe (CIPP) methods and in accordance with ASTM F1216, ASTM F1743, ASTM F3541, or ASTM F2599. Main and lateral cure-in-place rehabilitation of building sewer and sewer service lateral pipe and their connections to the main sewer pipe shall be in accordance with ASTM F2561. Hydrophilic rings or gaskets in cure-in-place rehabilitation of building sewer piping and sewer service laterals shall be in accordance with ASTM F3240 to ensure water tightness and elimination of ground water penetration